Object Type:	PID

Description:	PID (proportional, integral, derivative) feedback controller

Author:		M. Nelson, Caltech (4/89)

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	PID_type  [in src/user/user_struct.h]

Size:		144 bytes

Fields:		cmd          inputs the command (desired) value of sns

sns          inputs the sensed (measured) value

output       = gain*(e + tau_d*de/dt + 1/tau_i * e_integral)

gain         determines the sensitivity of the controller

tau_i        integration time constant, typically = dt

tau_d        derivative time constant, typically = dt/4

saturation   sets permissible range of output

e            internal variable (error = cmd-sns)

e_integral   internal variable (integral of e dt)

e_deriv      internal variable

e_previous   internal variable

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	PIDcontroller  [in src/user/PID.c]

Classes:	device

Actions:	INIT

PROCESS

RESET

Messages:	CMD command

SNS sense

GAIN gain

------------------------------------------------------------------------------

Notes:

The PID is used for feedback control of dynamical systems.  For example, in

a voltage clamp circuit it is used with a pulsegen, RC, and diffamp device

to provide the required injection current to maintain the membrane voltage

at the desired level.  The RC-filtered command voltage is scaled by the

diffamp and then presented to the PID with a CMD message.  The output is

sent to a compartment with an INJECT message, and the resulting Vm in the

compartment is fed back to the PID with a SNS message.  A higher gain gives

a better clamp, but requires a smaller integration time step to avoid

numerical instability.

For more information on PID controllers see: Franklin, Powel, Emami-Naeini

(1986) "Feedback Control of Dynamic Systems", Addison-Wesley, pp. 99-106.

Example:	Scripts/squid/squid_electronics.g

See also: